2010 Chevrolet Camaro LS

2010 SUSPENSION Front Suspension - Camaro

Fig. 4: Front Wheel Bearing & Hub Courtesy of GENERAL MOTORS CORP.

| Callout | Component Name |
| --- | --- |
| Preliminary Procedures - Raise and support the vehicle. Refer to Lifting and Jacking the Vehicle . - Remove the tire and wheel. Refer to Tire and Wheel Removal and Installation . - Remove the wheel speed sensor. Refer to Front Wheel Speed Sensor Replacement . - Remove the front brake rotor. Refer to Front Brake Rotor Replacement (LLT) or Front Brake Rotor Replacement (L99, LS3) . |  |
| 1 | Front Wheel bearing Bolt (Qty: 3) CAUTION: Refer to Fastener Caution . Tip: If needed, remove the front brake splash shield. Refer to Front Brake Shield Replacement to gain access to the front wheel bearing/hub bolts. Tighten: 108 N.m (79 lb ft) |
| 2 | Front Wheel Bearing/Hub Assembly Tip: Use the appropriate tool to separate the wheel bearing/hub from the wheel drive shaft. |

STEERING KNUCKLE REPLACEMENT