2010 Chevrolet Camaro LS

2010 SUSPENSION Rear Suspension - Camaro

2010 SUSPENSION

Rear Suspension - Camaro

# SPECIFICATIONS

FASTENER TIGHTENING SPECIFICATIONS

| Application | Specification |  |
| --- | --- | --- |
|  | Metric | English |
| Adjustable Link Bolt to Knuckle | 140 N.m | 103 lb ft |
| Adjustable Link Adjuster Nut | 115 N.m | 85 lb ft |
| Lower Control Arm Adjuster Nut | 115 N.m | 85 lb ft |
| Lower Control Arm to Knuckle Bolt |  |  |
| - First Pass | 40 N.m | 30 lb ft |
| - Final Pass | + 120° |  |
| Lower Rear Shock Absorber Nut |  |  |
| - First Pass | 80 N.m | 59 lb ft |
| - Final Pass | + 120° |  |
| Rear Shock Absorber Nut | 45 N.m | 33 lb ft |
| Rear Stabilizer Shaft Insulator Clamp Bolts | 22 N.m | 16 lb in |
| Rear Stabilizer Shaft Link Nuts | 49 N.m | 36 lb ft |
| Rear Wheel Bearing and Hub Bolts | 115 N.m | 85 lb ft |
| Trailing Arm to Frame Bolt | 100 N.m | 74 lb ft |
| Trailing Arm to Knuckle Bolts |  |  |
| - First Pass | 40 N.m | 30 lb ft |
| - Final Pass | + 120° |  |
| Upper Control Arm to Bushing Bolt | 160 N.m | 118 lb ft |
| Upper Control Arm to Knuckle Bolts |  |  |
| - First Pass | 60 N.m | 44 lb ft |
| - Final Pass | + 90° |  |
| Upper Control Arm to Frame Bolt |  |  |
| - First Pass | 80 N.m | 59 lb ft |
| - Final Pass | + 120° |  |
| Upper Control Arm to Bushing to Frame Bolt |  |  |
| - First Pass | 50 N.m | 37 lb ft |
| - Final Pass | + 120° |  |
| Upper Rear Shock Absorber Bolt | 58 N.m | 43 lb ft |

# REPAIR INSTRUCTIONS

REAR WHEEL BEARING AND HUB REPLACEMENT

Fig. 1: Rear Wheel Bearing & Hub Courtesy of GENERAL MOTORS CORP.

| Callout | Component Name |
| --- | --- |
| Preliminary Procedures - Raise and support the vehicle. Refer to Lifting and Jacking the Vehicle . - Remove the tire and wheel assembly. Refer to Tire and Wheel Removal and Installation . - Remove the rear brake rotor. Refer to Rear Brake Rotor Replacement (LLT) or Rear Brake Rotor Replacement (L99, LS3) . - Remove the rear wheel speed sensor. Refer to Rear Wheel Speed Sensor Replacement . - Separate the wheel drive shaft from the wheel bearing/hub assembly. |  |
| 1 | Rear Wheel Bearing and Hub Bolt (Qty: 3) CAUTION: Refer to Fastener Caution . Tighten: 115 N.m (85 lb ft) |
| 2 | Rear Wheel Bearing and Hub |

KNUCKLE REPLACEMENT